SDC Automation

SDC PLC Programming Standard — Draft for Critique

Written by the AI code-generation agent from the V4.2 standard templates and revision history. Purpose: critique this. Anything wrong, missing, or overstated is a gap in what the agent believes — correct it and the generated code follows.

# 1. Architecture

- One program per station:

S{nn}_{Name}   — context programs shared machine-wide: Supervisor, Alarms, Tracking, Recipe (V4.2)

- Routines: R00_Main (JSRs only) → R01_Inputs → R02_StateTransitions → R03_StateLogic → R04/R05_{Axis}Servo → R20_Alarms

- Axes are program-scope iq_{Axis} InOut parameters bound to controller axes via ParameterConnections

# 2. State engine

- State_Engine_128Max; transitions are MOVE(n, Control.StateReg); logic reads one-hot Status.State[n]

- State map:

0 SafetyStop · 1 Manual · 2 AutoNotReady · 3 AutoReady · 4–31 sequence (+3) · 99 Lockout · 100–124 init · 127 Faulted

- Safety stop and fault snapshot the restartable state (ONS + LIMIT 4–98 + MOVE to RestartState); state 2 is the recovery hub

# 3. State transitions (R02)

- One commented rung per target state; sequence-first ordering, mode/safety overrides at the bottom so they win the scan

- Every sequence transition gated XIC(SS_OK)

- Conditions test confirmations, not commands: servo = MAM.PC + InPos; pneumatic = derived position bits; sensorless = timer .DN

- Timers live in R01; R02 tests only .DN — no TON inside transition rungs

- Wideband early advance ([PC+InPos , IP+InPosWide]) where the next motion cannot collide with the settling axis

- DryRun bypasses part-dependent conditions; PartStarted jump states where applicable

# 4. State logic (R03)

- Deliberately thin: status bits, part-started latch, device outputs. No motion instructions here.

- Canonical output shape (seal-in, OTE not OTL):

[auto: set-state OR hold-unless-clear-state] OR [State[1] manual: HMI_Momentary set/clear] → OTE(q_Output)

- Part tracking: latch Attempt at pick/process confirm, Success at place/process complete, into \Tracking.p_Data.Nest[n].PartStatus.Station[StaNum]

# 5. Servo axes (R04/R05 — fixed per-axis module)

- ~22 rungs per axis: Ready → AutoEnable → Permissive → MSO → MSF → fault reset (MAFR/MASR) → jog → inch → homing (MAH or AOI_TorqueHome) → manual + auto param staging → single MAM → MAS quick-stop on permissive loss → torque limits → feedback → computed max accel/decel → position monitors

- Positions in HMI_{Axis}.Parameters.Positions[n] — stable indices, never reshuffled; confirmation via AOI_RangeCheck .InPos/.InPosWide

- MAM gated by ServoActionStatus + AxisHomedStatus + {Axis}Permissive; cross-axis permissives are mandatory (e.g., X moves only with Z at Clear, confirmed)

# 6. Pneumatic devices (R01 derivations)

- 2-sensor: XIC(sensor) XIO(oppositeSensor) XIC(output) XIO(oppositeOutput) → OTE(Moved)

- 1-sensor: sensed direction as above; blind direction inferred XIO(sensor) + outputs + TON

- Sensorless (grippers, retainers): output + delay timer → derived Opened/Closed bits; downstream logic uses derived bits only

- AOI_Debounce on DigitalSensor inputs — not on pneumatic position sensors

# 7. Faults, alarms, warnings (R20)

- Sequence timeouts via the state engine: [state] MOVE(ms, Control.FaultTime) + XIC(Status.TimeoutFlt), latched until FaultReset

- Pneumatic motions get per-direction watchdogs (output-on/confirm-off) plus both-sensors-true misconfiguration checks

- Two banks — ServoAlarm[] (drive fault w/ DTOS+CONCAT code, lost-home, quickstop cause) and Alarm[] (sequence); two ProgramAlarmHandler instances, outputs OR’d

- Runtime messages station-prefixed: CONCAT(g_StationList[StaNum], AlarmList[n], Message)

- “Waiting for part” style conditions are warnings driving q_Pause, not faults (V4.0 change); spare alarm slots held with AlwaysOff so indices stay stable

# 8. Machine coordination

- Station → indexer contract: q_StationComplete (cycle done), q_ActuatorsSafe (clear to index), q_Pause (hold dial, no fault)

- Indexer: dwell = AND of all stations’ complete bits; incremental MAM index; on-station = MOD nest math within tolerance; shot pin interlocked both directions

- Supervisor supplies q_ManualMode, q_SafetyOK, q_FaultReset, q_CycleStartLatch, q_CycleStopped

# 9. Naming (full words, no abbreviations)

| Prefix | Meaning | Example |
| --- | --- | --- |
| q_ | output / command | q_CloseGripper |
| i_ | physical input | i_PartPresent |
| p_ | published value / signal | p_CycleTime |
| iq_ | InOut parameter (axes) | iq_XAxis |
| HMI_ | HMI-facing structure | HMI_XAxis |
| a{nn}_ | controller-scope axis | a02_S01PNPXAxis |
| {Axis}_{Instr} | motion instruction tag | XAxis_MAM, ZAxis_MSO |

- Range-check instances named for the position: XAxisPick, ZAxisClear

# 10. Application-specific insertion points

- Every spot an engineer must complete is marked XIC(g_MachineBasic.AlwaysOff) with a “*Replace always off bits with real conditions” comment

- A generated program is a starting point a CE reviews and completes — never a black box

Questions for the Controls Leads

The agent generates from the standard above. These are the gaps it cannot resolve from the template files alone.

1. Jason’s finding first: an action commanded in the wrong state (Z move during gripper close) in the generated PnP — which rung exactly? That case becomes a permanent automated self-check.

2. Acceptance test: what makes generated code trustworthy enough to run on a machine, beyond importing clean?

3. Wideband policy: which transitions may early-advance, stated as one rule? (Current guess: only when the next motion cannot collide with the settling axis.)

4. Manual servo speeds: manual/jog rungs read Accel[0]/Decel[0], shared with auto. Intentional, or should manual have its own indices?

5. Init-with-part: templates re-enter mid-sequence when a part is gripped at power-up (124 + GripperClosed). What is the general rule for choosing the re-entry state on a new station?

6. Fault vs warning: the rule for which conditions stop the machine vs raise q_Pause? “Waiting for upstream” = warning — what else?

7. q_ActuatorsSafe: the precise definition per station type — all actuators home, or a defined safe subset per application?

8. Homing: when torque-home (AOI_TorqueHome) vs MAH? Who decides, and what marks a station as requiring homing before auto?

9. HMI_Toggle vs Tracking OpStatus: V4.2 sources Lockout/DryRun/SS from \Tracking OpStatus. Is HMI_Toggle officially dead in new programs?

10. Ask vs decide: before generating, the agent asks a short list (devices, transitions, timeouts, handshakes). What must ALWAYS be asked, and what may it default?

Also requested: export the context programs (Supervisor, Alarms, Tracking, Recipe, MainTask) as L5X to the template folder — station templates reference them, but the agent has not seen their contents.