Before the next version — 6 things to confirm

Servo PNP — one sentence each, a checkmark or a line settles it.

1.  When one state moves X and Z at the same time, “Z parked at Retract” can’t be the X permissive — how do you handle the X permissive during a simultaneous X+Z move (safe-elevation window, and is it the same value as the blend-start clearance)?

2.  In-position checks: one AOI_RangeCheck instance per named position (transition points included), never one generalized at-target check.

☐  Agree        ☐  Disagree — note:  ________________________________________

3.  An axis not homed at cycle start is a fault — station exits to 127 and the operator homes in manual — never a Severity-1 warning/pause.

☐  Agree        ☐  Disagree — note:  ________________________________________

4.  A two-speed axis stages the default profile unconditioned first, then the slow profile as a state-keyed branch after it in the same Auto Mode rung (last write wins).

☐  Agree        ☐  Disagree — note:  ________________________________________

5.  After a stop with a part in the gripper, the station resumes into the sequence toward place (the state-2 resume branch) — it never enters init 100 while holding a part.

☐  Agree        ☐  Disagree — note:  ________________________________________

6.  Alarms are one per motion with a distinct operator message (“Z Axis Failed To Reach Pick…”), even when that means nine or more alarm rungs.

☐  Agree        ☐  Disagree — note:  ________________________________________

Name / date:  ______________________________