JARVIS Machine-Level Code Draft — Review Packet

First-draft SDC tier-3 (full-machine) PLC code for controls lead critique  ·  August 19, 2026  ·  Dan Belliveau / JARVIS

File: C:\SDC-StateLogic\machine-code-draft\SDC_MachineLevel_JARVIS_Draft.L5X — 6 machine-level programs, ~147 rungs, validator: PASSED (no unresolved references, all state numbers legal). Machine shape: indexing dial (S00 indexer templates); linear and robot-cell differences flagged in rung comments. Every non-evidenced structure carries a "JARVIS INFERENCE:" rung comment so critique can be targeted.

# 1. What was generated, and what it rests on

| Program | Contents | Evidence basis |
| --- | --- | --- |
| Supervisor | Mode management (manual/auto), SafetyOK aggregation, cycle start/stop latch with CycleStopping→CycleStopped, fault-reset distribution, q_MachineStopReason (V4.1), light stack + horn via reconstructed LightStackControl AOI, machine cycle time + 25/100-cycle moving averages, g_MachineBasic/g_CPUDateTime housekeeping (R10_Global), 3 alarms + handler. 6 routines, 61 rungs. | INFERRED. Only the 5-output station contract (q_ManualMode/q_SafetyOK/q_FaultReset/q_CycleStartLatch/q_CycleStopped) and q_MachineRunning/q_MachineFaultActive/q_MachineStopReason names are evidenced (template context refs + revision history). Cycle-time rungs verbatim from X-drive Rungs\Machine Cycle Time. |
| Alarms | p_ProgramID zeroed per scan, machine fault/warning aggregation, active-list corruption clear, PV5500 popup 90/91 handling, history display navigation, "fault that stopped the machine" capture, TopAlarms call (top-5 since reset + per shift). 16 rungs. | EVIDENCE. Transcribed from the real Alarms program export (X-drive, V2.2 era); station contact lineup updated from \StateMachine/\S03 to the V4.2 template programs. SIZE→constant substitution flagged in-rung. |
| Tracking | p_Data (TrackingData UDT) + p_NestQty owner, OpStatus distribution (Lockout/DryRun/SingleStep per station, nest lockout), nest record lifecycle (clear on finished-nest return to load), good/reject part pulses, batch counter → cycle-stop request. 4 routines, 18 rungs. | INFERRED. UDT structure reverse-engineered from every \Tracking.p_Data access path in the V4.2 templates; PerformData pinned exactly by the StationPerformance AOI. All logic is inference — no export exists. |
| HMI | Screen/popup arbitration for PV5500 (popup queue, hidden-popup resend, 9999/8888 handshake) + q_CleanoutModeEnabled. | EVIDENCE. Verbatim from X-drive HMI export; cleanout rung added (V4.2 S01 references it; old export lacks it). |
| Production | Lifetime + per-shift counts, runtime/downtime, GoodRate/PPM/PerfectRunTotal, OEE AOI calls, shift calc (07:00/15:00/23:00), press-and-hold resets. 35 rungs. | EVIDENCE + V4.1 deltas. Base transcribed from X-drive export; the ten V4.1 changes applied from one-line revision notes and marked JARVIS INFERENCE. |
| Recipe | Numbered recipe slots (Name + 16 REAL parameters), HMI load/save handshake, load blocked while cycle running, distribution placeholder rung. | PURE INFERENCE. V4.2 adds "Recipe program + PanelView 5310 example"; nothing exported, nothing referenced by stations. |

Also in the file: the 11 evidenced UDTs (AlarmData, MachineBasic, CPU_TimeDate, StateLogic*, ProductionData w/ V4.1 members, TopAlarmsDT…), 7 inferred Tracking/Recipe UDTs, 8 AOIs (interfaces verified against X-drive/template definitions; ProgramAlarmHandler, State_Engine_128Max, TopAlarms, OEE, MovingAverage, AOI_Debounce as import-over stubs; CPU_TimeDate_wJulian with real logic; LightStackControl fully reconstructed), controller tags (g_MachineBasic, g_CPUDateTime, g_StationList[21], performance presets), context stubs for S00_IndexerSP/S01_PartLoad/S05_ServoPNP, and a MainTask schedule with ordering rationale.

## MainTask order (inference — rationale in the task description)

Supervisor → S00_IndexerSP → S01_PartLoad → S05_ServoPNP → Tracking → Alarms → Production → HMI → Recipe.  Supervisor decides before stations act; S00 recomputes NestNum before stations consume it; Tracking sees this-scan latches; Alarms aggregates after everyone runs. Linear: swap in S00_IndexerNoSP (revision history: NoSP doubles as the linear indexer). Robot cell: no S00 — robot interface programs take its slot.

# 2. JARVIS INFERENCE items — confirm or correct

- Supervisor state map — 0 Safety Stop / 1 Manual / 2 Idle Not Ready / 3 Idle Ready / 4 Cycle Running / 7 Cycle Stopping / 10 Cycle Stopped / 127 Fault, mirroring the station grammar. Real Supervisor numbering unknown.

- Safety OK aggregation — safety relay + air pressure inputs only. What belongs in the standard aggregation (gates, light curtains, GuardLogix status)?

- Cycle start latch conditions — idle-ready + StartOK + no faults; warnings do NOT block a start (stop-less bias).

- Cycle unlatch on any station fault — draft stops the whole machine on a single station fault. Alternative: pause the dial and let the faulted station recover in place.

- q_MachineStopReason code map — 1 Operator / 2 Manual / 3 Safety / 4 Fault / 5 Batch Complete, cleared on new start (V4.1 rungs 13-14 reconstructed).

- q_CycleStopped / q_MachineRunning state sets — running includes Cycle Stopping so the Alarms stop-fault capture can see the fault that ended the cycle.

- LightStackControl AOI — full reconstruction: green solid run, amber manual/flash warning, red flash fault / solid safety, cycle-start light flashes when ready, horn until HornOverride.

- g_MachineBasic + g_CPUDateTime ownership — housekeeping (AlwaysOn/Off, PowerUpCP, flashers, clock) placed in Supervisor R10_Global; no export shows where SDC actually drives these.

- Tracking UDT family — TrackingData / Station[21] / Nest[21] structure reverse-engineered from access paths; dimension 21 and 1-based indexing inferred; p_NestQty = 20.

- OpStatus distribution — HMI toggle words (bit = station number) mapped onto p_Data.Station[n].OpStatus Lockout/DryRun/SingleStep. Actual HMI write mechanism unknown.

- Nest record clearing — Tracking clears Nest[].PartStatus when a finished nest re-arrives at the load station; incomplete parts survive for fault recovery. Nothing in any template clears these bits, so someone must.

- Batch counter placement — revision history says only "Added batch counter" (V4.1); drafted in Tracking, counting good-part pulses, requesting a cycle stop (not a fault) at target.

- Good/Reject part triggers — Tracking publishes q_GoodPartPulse / q_RejectPartPulse into the Production ***Replace*** placeholders, keyed off unload-station Success/Failure.

- V4.1 Production changes — ElapsedSec/ElapsedMin/PPM/PerfectRunTotal formulas and fault-only downtime condition reconstructed from one-line revision notes.

- Recipe program — entirely inferred: 8 recipe slots of Name + 16 generic REAL parameters, HMI load/save handshake, load blocked unless cycle stopped, distribution rung left as engineer placeholder.

- Cleanout mode — q_CleanoutModeEnabled drafted as a plain HMI toggle with no interlock; S01 consumes it to stop loading new parts.

- MainTask scan order — Supervisor, S00, S01, S05, Tracking, Alarms, Production, HMI, Recipe (rationale in the task description).

- Machine cycle time source — IndexComplete one-shot derived from S00 p_OnStation rising edge feeding the evidenced cycle-time rung pair.

# 3. Questions for the controls leads

- How do part tracking and fault handling differ across linear, indexing-dial, and robot-cell machines? On a dial, S00 owns NestNum rotation and ActuatorsSafe gates the index. What replaces NestNum bookkeeping on a linear transfer (pallet ID? shift register?) and in a robot cell (no nests at all?), and does the "fault that stopped the machine" capture work the same way?

- Should one station fault stop the whole machine cycle, or should the Supervisor pause the dial (like the indexer pause states 40/42) and let the faulted station recover — which reading matches "machines that stop less"?

- Who clears Nest[].PartStatus and when? Stations only latch bits and S00 computes a NestsShiftedPulse it never publishes. Draft clears on return-to-load-with-unload-complete; is that right?

- Where do the OpStatus bits get written? Does the HMI write p_Data.Station[n].OpStatus bits directly, or does a Tracking routine mediate them as drafted?

- What is the real Supervisor state map and does it use State_Engine_128Max at all, or is it plain interlock logic without a state engine?

- LightStackControl AOI interface — what are the actual inputs/outputs beyond Cycle Start Light, Green, Amber, Horn, and HornOverride, and what are the correct flash states?

- q_MachineStopReason — what are the actual reason codes and does anything besides Production/HMI consume it?

- Recipe program scope — does the standard Recipe program own servo position sets per part number, or only simple setpoints? How does the PanelView 5310 handshake work (load/save/verify)?

How to review: open the L5X in any text editor or import into Studio 5000 v37 — every inferred rung is grep-able by "JARVIS INFERENCE". Corrections can be marked directly against the numbered list above.