Fanuc Robot Startup Checklist

Standard Procedure — All SDC Fanuc Robots

Step 1 — Power On & Software Verification

Perform the following immediately after robot power-on:

- Verify ordered software is installed:

  - Navigate: Menu > Status > Type > Version ID > Next > Order FI...

  - Confirm all software options match the slip sheet that came with the robot.

- If software is NOT installed:

  - Use the black USB drive from Fanuc.

  - Load software via BIOS menu: hold F1 + F5 while power cycling.

  - Select Software Load USB.

  - DO NOT choose existing personality.

- After software is verified — take a backup immediately:

  - Perform an Image Backup.

  - Perform an All of the Above (AOA) backup.

  - Store both in the job folder, labeled as Initial Image and AOA.

Step 2 — Robot Config Menu Setup

Navigate to: Menu > Next > System > Type > Config

Set all values to match the table below:

| # | Setting | Required Value |
| --- | --- | --- |
| 1 | USE HOT START | FALSE |
| 2 | I/O POWER FAIL RECOVERY | Recover All |
| 3 | Cold Start AutoExec PRG | NULL |
| 4 | Hot Start AutoExec PRG | NULL |
| 5 | Hot Start Done Signal | DO[0] |
| 6 | Restore Selected PRG | TRUE |
| 7 | Enable UI Signals | TRUE |
| 8 | START for CONTINUE only | TRUE |
| 9 | CSTOPI for ABORT | TRUE |
| 10 | Abort all programs by CSTOPI | TRUE |
| 11 | PROD-START DEPEND ON PNS | FALSE |
| 12 | DETECT FAULT_RESET SIGNAL | FALL |
| 13 | USE PPABN SIGNAL | GROUPS |
| 14 | WAIT TIMEOUT | 10.00 SEC |
| 15 | RECEIVE TIMEOUT | 30.00 SEC |
| 16 | RETURN TO TOP OF PROGRAM | TRUE |
| 17 | Original Program Name {F1} | PRG |
| 18 | Original Program Name {F2} | MAIN |
| 19 | Original Program Name {F3} | SUB |
| 20 | Original Program Name {F4} | TEST |
| 21 | Original Program Name {F5} | ****** |
| 22 | Default Logical Command | DETAIL |
| 23 | Maximum of ACC Instruction | 100 |
| 24 | Minimum of AA Instruction | 0 |
| 25 | WJNT for Default Motion | ***** |
| 26 | Auto Display of Alarm Menu | FALSE |
| 27 | Force Message | ENABLE |
| 28 | Allow Force I/O in AUTO mode | TRUE |
| 29 | Allow Chg. Ovrd. in AUTO mode | TRUE |
| 30 | Signal to Set in AUTO Mode | DO[200] |
| 31 | Signal to Set in T1 Mode | DO[201] |
| 32 | Signal to Set in T2 Mode | DO[202] |
| 33 | Signal to Set if E-STOP | DO[203] |
| 34 | Set if Input Simulated | DO[204] |
| 35 | Set if Output Simulated | DO[205] |
| 36 | Sim. Input Wait Delay | 0.00 sec |
| 37 | Set if Sim. Skip Enabled | DO[0] |
| 38 | Set when Prompt Displayed | DO[0] |
| 39 | Output when Wait on Input | DETAIL |
| 40 | Signal if Override = 100 | DO[206] |
| 41 | Hand Broken | GROUPS |
| 42 | Remote/Local Setup | OP Panel Key |
| 43 | External I/O (ON: Remote) | DI[0] |
| 44 | UOP Auto Assignment | None |
| 45 | Multi Program Selection | TRUE |

Step 3 — I/O Setup

- Use the I/O config file DIOCFGSV.IO to set up Digital I/O and UOP assignments.

- Use the EDA config file EIPCFG to set up EDA:

  - Verify Connection 2 in the Ethernet IP config has word size set to Zero.

Step 4 — Ethernet Configuration

Navigate to: Setup > Type > Next > Host Comm > Enter > Port 1

- Assign the designated IP address per project network documentation.

Step 5 — Load Standard TP Programs

- Load all standard TP programs as required per project specifications.

- Confirm program names match the Original Program Name settings configured in Step 2 (F1–F4).

Notes / Sign-Off

| Completed By: | Date: |
| --- | --- |
| Notes: |  |